# Project Documentation: ESP32 Wi-Fi Robotic Arm

# 1. Project Overview

This project is a 4DOF (4 Degrees of Freedom) robotic arm controlled via a custom-built, responsive web interface. The system uses an ESP32 microcontroller operating as a standalone Wi-Fi Access Point, allowing any smartphone or computer to control the arm without needing an external router. The software features custom motion-smoothing algorithms, physical hardware overrides, and non-volatile EEPROM memory to save and playback automated movement sequences.

# 2. Hardware Specifications

Microcontroller: ESP32 Development Board
PWM Driver: PCA9685 16-Channel 12-bit I2C Servo Driver
Actuators: 5x Standard 50Hz Servos (4 DOF: Shoulder, Elbow, Base, and a dual-servo Gripper acting as a single synchronized axis)
Physical Frame: Custom handmade ice cream stick chassis
Power Supply: 12V (3x Li-ion cells in series) regulated down to 5.5V via a buck converter for the servos.
Hardware Interface:
    • Manual Gripper Override: Two tactile push buttons (Pins 25 & 26).
    • Status Indicators: Red (Empty) and Green (Holding Object) LEDs.

# 3. Wiring & Pin Mapping

| Component | ESP32 Pin / Connection | Description |
| --- | --- | --- |
| PCA9685 (SDA) | GPIO 21 | I2C Data Line |
| PCA9685 (SCL) | GPIO 22 | I2C Clock Line |
| Close Button | GPIO 25 | Closes the gripper manually (Input Pullup) |
| Open Button | GPIO 26 | Opens the gripper manually (Input Pullup) |
| Red LED | GPIO 13 | Indicates the gripper is empty |
| Green LED | GPIO 14 | Indicates the gripper is securely holding an object |
| Servo 0 (Shoulder) | PCA9685 Channel 0 | Controls DOF 1 |
| Servo 1 (Elbow) | PCA9685 Channel 1 | Controls DOF 2 |
| Servo 2 (Base) | PCA9685 Channel 2 | Controls DOF 3 |
| Servo 3 (Grip S1) | PCA9685 Channel 3 | Controls DOF 4 (Primary) |
| Servo 4 (Grip S2) | PCA9685 Channel 4 | Synchronized to Grip S1 |

# 4. Software Architecture & Features

## A. Networking & Web Server

The ESP32 broadcasts its own Wi-Fi network:
• SSID: RoboticArm_AP
• Password: 123456789
• Server Type: Asynchronous handling on Port 80.
• UI Client: A sleek, dark-mode capable HTML/CSS frontend is injected directly from the ESP32's memory. It features API request throttling (max 20 requests/sec) to ensure the ESP32 does not crash when UI sliders are moved rapidly.

## B. Kinematics & Motion Control

Instead of snapping violently to target positions, the code implements a custom acceleration and deceleration engine:
• Velocity Braking: Servos automatically slow down as they approach their target coordinates (currentDecelDist), preventing mechanical jolts to the wooden frame.
• Independent Axis Speeds: The Base (Axis 2) and the Arm (Axes 0, 1) can be assigned different max speeds dynamically from the web interface.
• Anti-Jolt Wake-Up: The wakeUpServos() function ensures that when the system transitions from idle to active, it calculates current physical positions before moving, preventing aggressive snapping.

## C. EEPROM Memory Automation

The arm can be programmed like an industrial robot:
• Capacity: Up to 50 unique positional frames can be saved directly to the ESP32's non-volatile 2048-byte EEPROM.
• Playback State Machine: Features MANUAL, AUTO_PLAY, PAUSED, and STOPPED modes.
• Looping: Sequences can be set to loop infinitely via the frontend toggle.
• Power Loss Safety: Positions are committed to EEPROM, so automation sequences are retained even if the battery dies or the system is rebooted.

# 5. System Calibration Limits

To protect the handmade frame from tearing itself apart, hard software limits have been programmed into the MIN_ANGLES and MAX_ANGLES arrays:
• Shoulder: 60° to 120°
• Elbow: 60° to 130°
• Base: 10° to 85°
• Gripper: 30° to 90° (Primary) / 0° to 180° (Secondary)

# 6. Setup & Usage Instructions

1. Power Up: Turn on the 12V battery supply (ensuring it passes through the 5.5V buck converter) and power the ESP32.
2. Connect: Open Wi-Fi settings on a smartphone/PC and connect to RoboticArm_AP.
3. Launch Dashboard: Open a web browser and navigate to the IP address printed in the Serial Monitor (typically 192.168.4.1).
4. Control: Use the Joint Control sliders to move the arm.
5. Program a Sequence: Move the arm to a desired position and click "Save Current Position". Repeat this to build a sequence. Click "Start" to watch the arm execute the automated playback.

# 7. Showcase & Presentation Notes

For visual demonstrations and DIY showcases, position the camera for a clear top-down table recording perspective. Ensure the custom ice cream stick chassis is well-lit to highlight the handmade mechanical design. Keep the 12V Li-ion battery pack and buck converter neatly cable-managed to the side of the frame so the focus remains on the smooth kinematic movements of the arm and the glowing status LEDs.